!

! Last configuration change at 06:41:07 UTC Sat Mar 2 2002

version 15.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname ap

!

!

logging rate-limit console 9

enable secret 5 $1$WknO$xC1ZDtGsf5hV.DabDZunh1

!

no aaa new-model

no ip cef

!

!

!

!

dot11 syslog

!

dot11 ssid societe8

authentication open

authentication key-management wpa version 2

guest-mode

wpa-psk ascii 0 progtr00

!

dot11 ssid societe8_1

authentication open

guest-mode

!

!

dot11 guest

!

!

!

username Cisco password 0 progtr00

!

!

bridge irb

!

!

!

interface Dot11Radio0

no ip address

!

encryption mode ciphers aes-ccm

!

ssid societe8

!

antenna gain 0

station-role root

bridge-group 1

bridge-group 1 subscriber-loop-control

bridge-group 1 spanning-disabled

bridge-group 1 block-unknown-source

no bridge-group 1 source-learning

no bridge-group 1 unicast-flooding

!

interface Dot11Radio1

no ip address

antenna gain 0

peakdetect

no dfs band block

channel dfs

station-role root

bridge-group 1

bridge-group 1 subscriber-loop-control

bridge-group 1 spanning-disabled

bridge-group 1 block-unknown-source

no bridge-group 1 source-learning

no bridge-group 1 unicast-flooding

!

interface GigabitEthernet0

no ip address

duplex auto

speed auto

bridge-group 1

bridge-group 1 spanning-disabled

no bridge-group 1 source-learning

!

interface BVI1

ip address 192.168.10.10 255.255.255.0

ipv6 address dhcp

ipv6 address autoconfig

ipv6 enable

!

ip default-gateway 192.168.20.1

ip forward-protocol nd

ip http server

no ip http secure-server

ip http help-path http://www.cisco.com/warp/public/779/smbiz/prodconfig/help/eag

!

!

bridge 1 route ip

!

!

!

line con 0

line vty 0 4

login local

transport input all

!

end

code encrypté:

enable

configure terminal

hostname ap

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

logging rate-limit console 9

enable secret progtr00

no aaa new-model

no ip cef

dot11 syslog

dot11 ssid societe8

authentication open

authentication key-management wpa version 2

guest-mode

wpa-psk ascii progtr00

exit

dot11 ssid societe8_1

authentication open

guest-mode

exit

dot11 guest

username Cisco secret progtr00

bridge irb

interface Dot11Radio0

no ip address

encryption mode ciphers aes-ccm

ssid societe8

antenna gain 0

station-role root

bridge-group 1

bridge-group 1 subscriber-loop-control

bridge-group 1 spanning-disabled

bridge-group 1 block-unknown-source

no bridge-group 1 source-learning

no bridge-group 1 unicast-flooding

exit

interface Dot11Radio1

no ip address

antenna gain 0

peakdetect

no dfs band block

channel dfs

station-role root

bridge-group 1

bridge-group 1 subscriber-loop-control

bridge-group 1 spanning-disabled

bridge-group 1 block-unknown-source

no bridge-group 1 source-learning

no bridge-group 1 unicast-flooding

exit

interface GigabitEthernet0

no ip address

duplex auto

speed auto

bridge-group 1

bridge-group 1 spanning-disabled

no bridge-group 1 source-learning

exit

interface BVI1

ip address 192.168.10.10 255.255.255.0

ipv6 address dhcp

ipv6 address autoconfig

ipv6 enable

exit

ip default-gateway 192.168.20.1

ip forward-protocol nd

ip http server

no ip http secure-server

ip http help-path http://www.cisco.com/warp/public/779/smbiz/prodconfig/help/eag

bridge 1 route ip

line con 0

line vty 0 4

login local

transport input all

exit

end

write memory

enable secret 5 $1$mERr$nF0jmztN0trm4eGrYRe21.

username Cisco secret 5 $1$mERr$nF0jmztN0trm4eGrYRe21.

NB: 5 indique qu'il s'agit d'un mot de passe chiffré en utilisant l'algorithme MD5.

$1$mERr$nF0jmztN0trm4eGrYRe21. est la valeur chiffrée du mot de passe.