!

version 12.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname S1

!

boot-start-marker

boot-end-marker

!

!

username societe08 privilege 15 secret 5 $1$EdmJ$bNEVxQwOL5w6SB4ufdRLC.

!

!

no aaa new-model

switch 2 provision ws-c3750v2-48ps

system mtu routing 1500

ip domain-name switch.rt

!

!

!

!

crypto pki trustpoint TP-self-signed-1829767296

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-1829767296

revocation-check none

rsakeypair TP-self-signed-1829767296

!

!

crypto pki certificate chain TP-self-signed-1829767296

certificate self-signed 01

3082023F 308201A8 A0030201 02020101 300D0609 2A864886 F70D0101 04050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 31383239 37363732 3936301E 170D3933 30333031 30303033

35345A17 0D323030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D31 38323937

36373239 3630819F 300D0609 2A864886 F70D0101 01050003 818D0030 81890281

8100E82E 522BBEE0 72EC7D54 75EE7854 2C694683 BB9775E0 84A05896 9533CFD3

384137A9 94AB57BB 02D193AB AEBEB009 E353366C 9507A1EA 78C4C761 38AA1A28

50A6E5C8 CFE2E76D D68948CD B161CD4D 6C597E1B CBBDD8A1 BB11BC9A 9910863A

914E2C6E 9E19A833 6486F60B 98940C63 7A93DA57 339264F9 53EA3D82 51DD9F6E

CEC50203 010001A3 67306530 0F060355 1D130101 FF040530 030101FF 30120603

551D1104 0B300982 07537769 7463682E 301F0603 551D2304 18301680 14BF8961

7749B83C F3EFC33D 223FA440 55E9DAEC AC301D06 03551D0E 04160414 BF896177

49B83CF3 EFC33D22 3FA44055 E9DAECAC 300D0609 2A864886 F70D0101 04050003

818100A0 F77823BD 7A933283 376BD577 663E96AE 6F0A4E78 BA706FF1 5C56C7D3

2335DBA5 5DB8855B 34739087 8CBF349A FE50A778 6AD85F50 BC109A91 15ED69DD

9AA1DCB3 67F9CBAD FAF335A3 E40E4133 B23071E7 303CCBE4 5651DFBD 7E5C6693

DA4D8E36 24E1C1B5 7226ABFA 50614A03 CD9BD227 71E03CB2 72475C29 42855E3A 0ECEA3

quit

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

vlan internal allocation policy ascending

!

ip ssh version 2

!

!

!

interface FastEthernet2/0/1

switchport access vlan 20

switchport mode access

!

interface FastEthernet2/0/2

!

interface FastEthernet2/0/3

switchport access vlan 40

switchport mode access

!

interface FastEthernet2/0/4

!

interface FastEthernet2/0/5

switchport access vlan 10

switchport mode access

!

interface FastEthernet2/0/6

!

interface FastEthernet2/0/7

!

interface FastEthernet2/0/8

!

interface FastEthernet2/0/9

switchport trunk encapsulation dot1q

switchport trunk allowed vlan 1-99

switchport mode trunk

!

interface FastEthernet2/0/10

!

interface FastEthernet2/0/11

switchport access vlan 10

switchport mode access

!

interface FastEthernet2/0/12

!

interface FastEthernet2/0/13

!

interface FastEthernet2/0/14

!

interface FastEthernet2/0/15

switchport access vlan 10

switchport mode access

!

interface FastEthernet2/0/16

!

interface FastEthernet2/0/17

switchport access vlan 10

switchport mode access

!

interface FastEthernet2/0/18

!

interface FastEthernet2/0/19

!

interface FastEthernet2/0/20

!

interface FastEthernet2/0/21

switchport access vlan 30

switchport mode access

!

interface FastEthernet2/0/22

!

interface FastEthernet2/0/23

switchport access vlan 10

switchport mode access

!

interface FastEthernet2/0/24

!

interface FastEthernet2/0/25

!

interface FastEthernet2/0/26

!

interface FastEthernet2/0/27

!

interface FastEthernet2/0/28

!

interface FastEthernet2/0/29

!

interface FastEthernet2/0/30

!

interface FastEthernet2/0/31

!

interface FastEthernet2/0/32

!

interface FastEthernet2/0/33

!

interface FastEthernet2/0/34

!

interface FastEthernet2/0/35

!

interface FastEthernet2/0/36

!

interface FastEthernet2/0/37

!

interface FastEthernet2/0/38

!

interface FastEthernet2/0/39

!

interface FastEthernet2/0/40

!

interface FastEthernet2/0/41

!

interface FastEthernet2/0/42

!

interface FastEthernet2/0/43

!

interface FastEthernet2/0/44

!

interface FastEthernet2/0/45

!

interface FastEthernet2/0/46

!

interface FastEthernet2/0/47

!

interface FastEthernet2/0/48

!

interface GigabitEthernet2/0/1

!

interface GigabitEthernet2/0/2

!

interface GigabitEthernet2/0/3

!

interface GigabitEthernet2/0/4

!

interface Vlan1

ip address 192.168.0.1 255.255.255.0

!

ip classless

ip http server

ip http secure-server

!

!

!

!

!

line con 0

line vty 0 4

login local

transport input ssh

transport output ssh

line vty 5 15

login

!

end